Part 3 – Certification Checklist of Minimum Standards of Living Conditions

(Please complete all relevant parts of the checklist with enclosure of supporting documents and photo record)

| Part 3A – SDUs (Certify item 1 to 8 for each SDU intended for recognition. If more than 1 SDU is to be certified, please separately fill in additional pages) |  |  |  |
| --- | --- | --- | --- |
| SDU（ [Room no. of SDU] ） |  |  |  |
| Item | Requirements of Minimum Standards of Living Conditions | Requirements complied with (If requirements are complied with, please indicate with a “P” sign. Please indicate “N/A” for items that are not applicable.) | Photo record of existing conditions / supporting documents must be provided (Please insert photo record at Part 4 and list the applicable supporting documents at Part 5. Please attach the above to this form of certification report. ) |
| 1 | Minimum internal floor area (IFA) |  |  |
| 1.1 | The IFA of the SDU is at least 8m2 (Refer to para. 4.2 of Code of Practice for relevant requirements) |  | Indicate the IFA of the SDU at the layout plan of the principal flat and SDUs in Part 2(a) |
| 2 | Minimum Height |  |  |
| 2.1 | The SDU has a height of at least 2.3m measured from floor to ceiling (Refer to para. 4.3 of Code of Practice for relevant requirements) |  | Photo record indicating the relevant height (Photo number: ) |
| 2.2 | The SDU has a height of at least 2m measured from floor to the underside of any beam (Refer to para. 4.3 of Code of Practice for relevant requirements) |  | Photo record indicating the relevant height (Photo number: ) |
| 3 | Fire safety |  |  |
| 3.1 | The SDU has access to at least 2 required staircases on that floor at all times (Not applicable if the principal building has only 1 required staircase for means of escape in case of fire as shown in the reference building plan) (Refer to para. 4.4.1(a) of Code of Practice for relevant requirements) |  |  |
| - Please certify item 3.2, 3.3, or 3.4 based on the construction and design of the SDU; Please indicate “N/A” for items that are not applicable. |  |  |  |
| 3.2 | The fire rated door at the original entrance of the principal flat is retained or replaced by a fire rated door with fire resisting construction equivalent to or better than the original fire rated door, and a shared corridor leading to the SDUs is provided: (Refer to para. 4.4.1(b) of Code of Practice for relevant requirements) |  |  |
|  | - The entrance of the SDU is provided with a self-closing door with smoke seal |  | Photo record of self-closing door with smoke seal (Photo number: ) One of the following supporting documents: Test / assessment report of the door with smoke seal Others (please specify): |
|  | - The walls between different SDUs are solid partition walls constructed of non-combustible materials with protection against smoke |  | Photo record of partition walls (Photo number: ) One of the following supporting documents: Test / assessment report of non-combustible partition walls Others (please specify):： |
| 3.3 | The fire rated door at the original entrance of the principal flat is removed or replaced by a door with fire resisting construction inferior to the original fire rated door, and a shared corridor leading to the SDUs is provided: (Refer to para. 4.4.1(c) of Code of Practice for relevant requirements) |  |  |
|  | - The entrance of the SDU is provided with a fire rated door of not less than 1 hour fire resisting construction, and such door is a self-closing door with smoke seal |  | Photo record of fire rated door (Photo number: ) One of the following supporting documents: Test / assessment report of fire rated door Others (please specify): |
|  | - The walls between different SDUs have not less than 1 hour fire resisting construction and with protection against smoke |  | Photo record of partition walls (Photo number: ) One of the following supporting documents: Test/ assessment report of partition walls on fire resisting construction Others (please specify): |
| 3.4 | The entrance door of the SDU gives direct access to the common corridor of the principal building: (Refer to para. 4.4.1(d) of Code of Practice for relevant requirements) |  |  |
|  | - The entrance of the SDU is provided with a fire rated door of not less than 1 hour fire resisting construction, and such door is a self-closing door with smoke seal |  | Photo record of fire rated door (Photo number: ) One of the following supporting documents: Test / assessment report of fire rated door Others (please specify): |
|  | - The walls between the common corridor of the principal building and the SDU and the walls between different SDUs have not less than 1 hour fire resisting construction and with protection against smoke |  | Photo record of partition walls (Photo number: ) One of the following supporting documents: Test / assessment report of partition walls on fire resisting construction Others (please specify): |
| 3.5 | If naked flame cooking facility is provided in the SDU, the facility is installed either in a separate enclosed kitchen that complies with the requirements of the Buildings Ordinance, or in an open kitchen approved by both the Buildings Department and the Fire Services Department. (Refer to para. 4.4.1(h) of Code of Practice for relevant requirements) |  | Photo record of kitchen (Photo number: ) One of the following supporting documents: Separate enclosed kitchen: Test / assessment report of partition walls and fire rated door for fire resisting construction or relevant approved building plan* (* delete as appropriate) Open kitchen: relevant approved building plan Others (please specify): |
| 3.6 | The SDU is provided with stand-alone fire detector (Refer to para. 4.4.2 of Code of Practice for relevant requirements) |  | Photo record of stand-alone fire detector (Photo number: ) Copy of Certificate of Fire Service Installation and Equipment (FS251) |
| 4 | Loading |  |  |
| 4.1 | The SDU subject to a load within its proper bearing capacity (Refer to para. 4.5 of Code of Practice for relevant requirements) |  | Structural assessment report |
| 5 | Separate Toilet |  |  |
| 5.1 | Within each SDU at least 1 enclosed and waterproofed toilet for exclusive use by its occupier is provided (Refer to para. 4.6(a) of Code of Practice for relevant requirements) |  | Photo record of separate toilet, its partition walls and door (Photo number: ) |
| 5.2 | The separate toilet is fitted with a watercloset, a lavatory basin, and a bath or shower (Refer to para. 4.6 (a) and (b) of Code of Practice for relevant requirements) |  | Photo record of watercloset, a lavatory basin, and a bath or shower (Photo number: ) |
| 5.3 | The separate toilet is waterproofed and does not cause water seepage nuisance to the units below or adjacent (Refer to para. 4.6(c) and (d) of Code of Practice for relevant requirements) |  | Following supporting documents (Please select as appropriate): Photo record of dye test (Photo number: ) Photo record of moisture monitoring with electronic moisture meter (Photo number: ) Apart from the above, other supporting documents to be provided (please specify): |
| 6 | Water Supply Point and Sink Outside Toilet |  |  |
| 6.1 | At least 1 water supply point and sink is provided outside the separate toilet (Refer to para. 4.7(a) of Code of Practice for relevant requirements) |  | Photo record of water supply point and sink (Photo number: ) |
| 6.2 | The area around the water supply point and sink is waterproofed and does not cause water seepage nuisance to the units below or adjacent (Refer to para. 4.7(b) and (c) of Code of Practice for relevant requirements) |  | Following supporting document (Please select as appropriate): Photo record of dye test (Photo number: ) Photo record of moisture monitoring with electronic moisture meter (Photo number: ) Apart from the above, other supporting documents to be provided (please specify): |
| 7 | Lighting and Ventilation |  |  |
| - Please certify item 7.1 or 7.2 based on the construction and design of habitable area in SDU; Please indicate “N/A” for items that are not applicable. |  |  |  |
| 7.1 | The habitable area in the SDU meets the requirements for natural lighting and ventilation as stipulated in regulations 30 to 33 of the Building (Planning) Regulations (Cap. 123F) |  | Photo record of window with dimensions (Photo number: ) |
| 7.2 | The habitable area in SDU (Refer to para. 4.8.2 of Code of Practice for relevant requirements) |  |  |
|  | - is fitted with at least 1 openable window that has a superficial area of glass of at least 0.1 m2 and provides adequate access to the external air |  | Photo record of window with glazing area (Photo number: ) |
|  | - is fitted with artificial lighting and mechanical means of ventilation |  | Photo record of artificial lighting and mechanical means of ventilation (Photo number: ) Ventilation assessment report for mechanical means of ventilation |
| - Please certify item 7.3 or 7.4 based on the construction and design of the separate toilet in the SDU; Please indicate “N/A” for items that are not applicable. |  |  |  |
| 7.3 | The separate toilet in the SDU meets the requirements for natural lighting and ventilation as stipulated in regulation 36 of the Building (Planning) Regulations (Cap. 123F) |  | Photo record of window with dimensions (Photo number: ) |
| 7.4 | The separate toilet is fitted with artificial lighting, mechanical means of ventilation and permanent ventilation (Refer to para. 4.8.3 of Code of Practice for relevant requirements) |  | Photo record of artificial lighting, mechanical means of ventilation and permanent ventilation (Photo number: ) Ventilation assessment report for mechanical means of ventilation |
| - Please certify item 7.5 or 7.6 based on the construction and design of the separate toilet in SDU; Please indicate “N/A” for items that are not applicable. |  |  |  |
| 7.5 | The kitchen in the SDU meets the requirements for natural lighting and ventilation in as stipulated in regulations 30 and 31 of the Building (Planning) Regulations (Cap. 123F) |  | Photo record of window with dimensions (Photo number: ) |
| 7.6 | The kitchen in the SDU (Refer to para. 4.8.2 of Code of Practice for relevant requirements) |  |  |
|  | - is fitted with at least 1 openable window that has a superficial area of glass of at least 0.1 m2 and provides adequate access to the external air |  | Photo record of window with glazing area (Photo number: ) |
|  | - is fitted with artificial lighting and mechanical means of ventilation |  | Photo record of artificial lighting and mechanical means of ventilation (Photo number: ) Ventilation assessment report for mechanical means of ventilation |
| 8 | Separate Water Meter and Electricity Meter |  |  |
| 8.1 | The SDU is provided with a separate water meter provided by the Water Supplies Department (Refer to para. 4.9.2 of Code of Practice for relevant requirements) |  | Photo record of separate water meter (Photo number: ) One of the following supporting documents (If applicable): water bills certificate of completion of installation of separate water meter |
| 8.2 | The SDU is provided with a separate electricity meter provided by a power company (Refer to para. 4.9.3 of Code of Practice for relevant requirements) |  | Photo record of separate electricity meter (Photo number: ) Following supporting document (If applicable): electricity bills |

Supplementary information (if any)